Supplement

XRF (X-Ray Fluorescence) measurements were performed with a Panalytical Axios Max WD-XRF spectrometer and data were evaluated using SuperQ5.0i/Omnian software at the faculty of Material Science and Engineering, Delft University of Technology. Sample preparation involved grinding to powder a small fraction of the collected rock samples and press 2.5 g of the powder into tablets for analysis. XRF results are shown in weight (%) using oxide compounds for each element. No correction for medium and film was applied. The table below shows the results of the XRF analysis as compound name, weight (%), and absolute error (%). CT scans of five rock samples were obtained at the Faculty of Civil Engineering and Geoscience at the Delft University of Technology. For each sample we scanned a cylinder of 2.5 mm diameter and 1.6 mm length. The resulting images have 2 micron resolution.

Table A1: XRF results

|  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 75.813 | 0.1 | SiO2 | 72.666 | 0.1 | SiO2 | 78.439 | 0.1 | SiO2 | 73.378 | 0.1 | SiO2 | 75.753 | 0.1 |
| CaO | 8.100 | 0.08 | CaO | 8.550 | 0.08 | CaO | 7.528 | 0.08 | CaO | 9.675 | 0.09 | Al2O3 | 7.793 | 0.08 |
| Al2O3 | 7.099 | 0.08 | Al2O3 | 7.726 | 0.08 | Al2O3 | 6.090 | 0.07 | Al2O3 | 6.784 | 0.08 | CaO | 7.472 | 0.08 |
| MgO | 3.803 | 0.06 | MgO | 3.642 | 0.06 | MgO | 3.218 | 0.05 | MgO | 4.751 | 0.06 | MgO | 3.513 | 0.06 |
| Fe2O3 | 2.200 | 0.04 | Fe2O3 | 3.426 | 0.05 | Fe2O3 | 2.031 | 0.04 | Fe2O3 | 2.563 | 0.05 | Fe2O3 | 2.225 | 0.04 |
| K2O | 1.269 | 0.03 | K2O | 1.582 | 0.04 | K2O | 1.079 | 0.03 | K2O | 1.263 | 0.03 | K2O | 1.382 | 0.04 |
| SO3 | 0.428 | 0.02 | SO3 | 0.827 | 0.03 | SO3 | 0.461 | 0.02 | SO3 | 0.418 | 0.02 | SO3 | 0.502 | 0.02 |
| TiO2 | 0.328 | 0.02 | TiO2 | 0.460 | 0.02 | P2O5 | 0.27 | 0.02 | TiO2 | 0.312 | 0.02 | TiO2 | 0.361 | 0.02 |
| P2O5 | 0.310 | 0.02 | NiO | 0.410 | 0.02 | NiO | 0.264 | 0.02 | P2O5 | 0.255 | 0.02 | P2O5 | 0.305 | 0.02 |
| Na2O | 0.220 | 0.01 | P2O5 | 0.290 | 0.02 | TiO2 | 0.251 | 0.02 | NiO | 0.200 | 0.01 | Na2O | 0.215 | 0.01 |
| NiO | 0.182 | 0.01 | Na2O | 0.154 | 0.01 | Na2O | 0.175 | 0.01 | Na2O | 0.199 | 0.01 | F | 0.156 | 0.01 |
| Cl | 0.038 | 0.006 | Cr2O3 | 0.073 | 0.008 | Cr2O3 | 0.05 | 0.007 | BaO | 0.048 | 0.007 | NiO | 0.150 | 0.01 |
| ZrO2 | 0.038 | 0.006 | ZrO2 | 0.053 | 0.007 | MnO | 0.031 | 0.005 | Cr2O3 | 0.039 | 0.006 | Cr2O3 | 0.035 | 0.006 |
| BaO | 0.034 | 0.006 | CuO | 0.051 | 0.007 | ZrO2 | 0.029 | 0.005 | MnO | 0.035 | 0.006 | ZrO2 | 0.035 | 0.006 |
| Cr2O3 | 0.03 | 0.005 | MnO | 0.03 | 0.005 | CuO | 0.023 | 0.005 | ZrO2 | 0.033 | 0.005 | MnO | 0.025 | 0.005 |
| MnO | 0.028 | 0.005 | Cl | 0.022 | 0.004 | Cl | 0.02 | 0.004 | Cl | 0.016 | 0.004 | CeO2 | 0.021 | 0.004 |
| CuO | 0.021 | 0.004 | SrO | 0.014 | 0.004 | PbO | 0.013 | 0.003 | SrO | 0.010 | 0.003 | Cl | 0.013 | 0.003 |
| PbO | 0.017 | 0.004 | ZnO | 0.012 | 0.003 | ZnO | 0.011 | 0.003 | CuO | 0.010 | 0.003 | CuO | 0.011 | 0.003 |
| ZnO | 0.016 | 0.004 | Rb2O | 0.008 | 0.003 | SrO | 0.009 | 0.003 | ZnO | 0.007 | 0.002 | SrO | 0.010 | 0.003 |
| SrO | 0.009 | 0.003 | Y2O3 | 0.003 | 0.002 | Rb2O | 0.004 | 0.002 | Rb2O | 0.005 | 0.002 | PbO | 0.007 | 0.003 |

|  | 6 |  |  | 7 |  |  | 8 |  |  | 9 |  |  | 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 75.765 | 0.100 | SiO2 | 73.934 | 0.100 | SiO2 | 66.114 | 0.100 | SiO2 | 66.273 | 0.100 | SiO2 | 68.003 | 0.100 |
| Al2O3 | 8.037 | 0.080 | Al2O3 | 9.424 | 0.090 | Al2O3 | 11.104 | 0.090 | Al2O3 | 11.665 | 0.100 | Al2O3 | 12.816 | 0.100 |
| CaO | 7.072 | 0.080 | CaO | 7.153 | 0.080 | CaO | 10.618 | 0.090 | CaO | 10.117 | 0.090 | CaO | 7.785 | 0.080 |
| MgO | 3.431 | 0.050 | MgO | 3.697 | 0.060 | MgO | 5.043 | 0.070 | MgO | 4.588 | 0.060 | MgO | 4.490 | 0.060 |
| Fe2O3 | 2.390 | 0.050 | Fe2O3 | 2.295 | 0.040 | Fe2O3 | 2.834 | 0.050 | Fe2O3 | 2.971 | 0.050 | Fe2O3 | 3.125 | 0.050 |
| K2O | 1.365 | 0.030 | K2O | 1.551 | 0.040 | K2O | 1.758 | 0.040 | K2O | 1.861 | 0.040 | K2O | 1.989 | 0.040 |
| SO3 | 0.527 | 0.020 | SO3 | 0.436 | 0.020 | SO3 | 1.095 | 0.030 | SO3 | 0.951 | 0.030 | TiO2 | 0.542 | 0.020 |
| TiO2 | 0.341 | 0.020 | TiO2 | 0.408 | 0.020 | TiO2 | 0.474 | 0.020 | TiO2 | 0.504 | 0.020 | P2O5 | 0.333 | 0.020 |
| P2O5 | 0.286 | 0.020 | P2O5 | 0.311 | 0.020 | P2O5 | 0.369 | 0.020 | P2O5 | 0.352 | 0.020 | Na2O | 0.247 | 0.010 |
| NiO | 0.223 | 0.010 | Na2O | 0.241 | 0.010 | Na2O | 0.307 | 0.020 | Na2O | 0.246 | 0.010 | F | 0.185 | 0.020 |
| Na2O | 0.217 | 0.010 | NiO | 0.191 | 0.010 | NiO | 0.104 | 0.010 | F | 0.202 | 0.020 | SO3 | 0.164 | 0.010 |
| F | 0.183 | 0.020 | F | 0.148 | 0.010 | MnO | 0.040 | 0.006 | NiO | 0.119 | 0.010 | NiO | 0.126 | 0.010 |
| Cr2O3 | 0.047 | 0.006 | ZrO2 | 0.034 | 0.005 | Cr2O3 | 0.028 | 0.005 | ZrO2 | 0.029 | 0.005 | MnO | 0.039 | 0.006 |
| ZrO2 | 0.031 | 0.005 | CuO | 0.033 | 0.005 | ZrO2 | 0.027 | 0.005 | Cr2O3 | 0.028 | 0.005 | Cr2O3 | 0.033 | 0.005 |
| Cl | 0.021 | 0.004 | Cr2O3 | 0.033 | 0.005 | Cl | 0.019 | 0.004 | MnO | 0.024 | 0.005 | ZrO2 | 0.026 | 0.005 |
| MnO | 0.018 | 0.004 | MnO | 0.029 | 0.005 | SrO | 0.015 | 0.004 | SrO | 0.016 | 0.004 | CeO2 | 0.022 | 0.004 |
| CuO | 0.011 | 0.003 | CeO2 | 0.028 | 0.005 | ZnO | 0.009 | 0.003 | Cl | 0.014 | 0.004 | SrO | 0.016 | 0.004 |
| SrO | 0.010 | 0.003 | Cl | 0.017 | 0.004 | Rb2O | 0.007 | 0.003 | CuO | 0.011 | 0.003 | ZnO | 0.014 | 0.004 |
| PbO | 0.008 | 0.003 | SrO | 0.012 | 0.003 | CuO | 0.006 | 0.002 | ZnO | 0.010 | 0.003 | CuO | 0.014 | 0.004 |
| ZnO | 0.006 | 0.002 | ZnO | 0.012 | 0.003 | PbO | 0.006 | 0.002 | Rb2O | 0.008 | 0.003 | Cl | 0.013 | 0.003 |

|  | 11 |  |  | 12 |  |  | 13 |  |  | 14 |  |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 65.356 | 0.100 | SiO2 | 63.704 | 0.100 | SiO2 | 63.510 | 0.100 | SiO2 | 63.940 | 0.100 | SiO2 | 63.443 | 0.100 |
| Al2O3 | 12.899 | 0.100 | Al2O3 | 14.305 | 0.100 | Al2O3 | 14.813 | 0.100 | Al2O3 | 17.023 | 0.100 | Al2O3 | 15.675 | 0.100 |
| CaO | 9.219 | 0.090 | CaO | 8.972 | 0.090 | CaO | 7.886 | 0.080 | CaO | 5.766 | 0.070 | CaO | 7.508 | 0.080 |
| MgO | 4.680 | 0.060 | MgO | 4.613 | 0.060 | MgO | 4.956 | 0.070 | MgO | 4.700 | 0.060 | MgO | 4.855 | 0.060 |
| Fe2O3 | 3.264 | 0.050 | Fe2O3 | 3.500 | 0.060 | Fe2O3 | 3.887 | 0.060 | Fe2O3 | 3.710 | 0.060 | Fe2O3 | 3.673 | 0.060 |
| K2O | 2.010 | 0.040 | K2O | 2.150 | 0.040 | K2O | 2.247 | 0.040 | K2O | 2.456 | 0.050 | K2O | 2.260 | 0.040 |
| SO3 | 0.940 | 0.030 | SO3 | 1.100 | 0.030 | SO3 | 0.954 | 0.030 | SO3 | 0.837 | 0.030 | SO3 | 0.866 | 0.030 |
| TiO2 | 0.543 | 0.020 | TiO2 | 0.576 | 0.020 | TiO2 | 0.567 | 0.020 | TiO2 | 0.632 | 0.020 | TiO2 | 0.568 | 0.020 |
| P2O5 | 0.348 | 0.020 | P2O5 | 0.354 | 0.020 | P2O5 | 0.344 | 0.020 | P2O5 | 0.304 | 0.020 | P2O5 | 0.345 | 0.020 |
| Na2O | 0.241 | 0.010 | Na2O | 0.231 | 0.010 | F | 0.319 | 0.020 | Na2O | 0.223 | 0.010 | F | 0.266 | 0.020 |
| F | 0.183 | 0.020 | F | 0.202 | 0.020 | Na2O | 0.221 | 0.010 | F | 0.175 | 0.020 | Na2O | 0.195 | 0.010 |
| NiO | 0.153 | 0.010 | NiO | 0.130 | 0.010 | NiO | 0.136 | 0.010 | NiO | 0.067 | 0.008 | NiO | 0.178 | 0.010 |
| Cr2O3 | 0.035 | 0.006 | ZrO2 | 0.024 | 0.005 | MnO | 0.031 | 0.005 | MnO | 0.027 | 0.005 | Cr2O3 | 0.039 | 0.006 |
| MnO | 0.034 | 0.006 | Cr2O3 | 0.024 | 0.005 | Cr2O3 | 0.031 | 0.005 | CeO2 | 0.026 | 0.005 | MnO | 0.028 | 0.005 |
| ZrO2 | 0.025 | 0.005 | MnO | 0.024 | 0.005 | ZrO2 | 0.022 | 0.004 | Cr2O3 | 0.021 | 0.004 | ZrO2 | 0.021 | 0.004 |
| SrO | 0.017 | 0.004 | CeO2 | 0.023 | 0.005 | SrO | 0.017 | 0.004 | ZrO2 | 0.020 | 0.004 | SrO | 0.016 | 0.004 |
| Cl | 0.015 | 0.004 | SrO | 0.018 | 0.004 | ZnO | 0.014 | 0.004 | SrO | 0.015 | 0.004 | Rb2O | 0.013 | 0.003 |
| Rb2O | 0.010 | 0.003 | Cl | 0.014 | 0.004 | Cl | 0.012 | 0.003 | ZnO | 0.014 | 0.004 | ZnO | 0.011 | 0.003 |
| ZnO | 0.010 | 0.003 | ZnO | 0.012 | 0.003 | Rb2O | 0.011 | 0.003 | Rb2O | 0.013 | 0.003 | Cl | 0.011 | 0.003 |
| CuO | 0.008 | 0.003 | Rb2O | 0.010 | 0.003 | CuO | 0.010 | 0.003 | Cl | 0.012 | 0.003 | V2O5 | 0.010 | 0.003 |

|  | 16 |  |  | 17 |  |  | 18 |  |  | 19 |  |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 65.766 | 0.100 | SiO2 | 66.604 | 0.100 | SiO2 | 35.845 | 0.100 | SiO2 | 65.844 | 0.100 | SiO2 | 74.465 | 0.100 |
| Al2O3 | 15.442 | 0.100 | Al2O3 | 14.691 | 0.100 | CaO | 34.835 | 0.100 | Al2O3 | 15.635 | 0.100 | Al2O3 | 10.444 | 0.090 |
| CaO | 5.970 | 0.070 | CaO | 6.073 | 0.070 | Al2O3 | 9.984 | 0.090 | CaO | 5.757 | 0.070 | CaO | 4.681 | 0.060 |
| MgO | 4.356 | 0.060 | MgO | 4.801 | 0.060 | MgO | 9.231 | 0.090 | MgO | 4.417 | 0.060 | MgO | 3.880 | 0.060 |
| Fe2O3 | 3.617 | 0.060 | Fe2O3 | 3.361 | 0.050 | Fe2O3 | 6.032 | 0.070 | Fe2O3 | 3.438 | 0.050 | Fe2O3 | 2.656 | 0.050 |
| K2O | 2.225 | 0.040 | K2O | 2.179 | 0.040 | K2O | 2.069 | 0.040 | K2O | 2.222 | 0.040 | K2O | 1.529 | 0.040 |
| SO3 | 0.883 | 0.030 | SO3 | 0.613 | 0.020 | SO3 | 0.671 | 0.020 | SO3 | 0.888 | 0.030 | SO3 | 0.679 | 0.020 |
| TiO2 | 0.587 | 0.020 | TiO2 | 0.561 | 0.020 | TiO2 | 0.537 | 0.020 | TiO2 | 0.585 | 0.020 | TiO2 | 0.454 | 0.020 |
| P2O5 | 0.344 | 0.020 | P2O5 | 0.340 | 0.020 | P2O5 | 0.224 | 0.010 | P2O5 | 0.354 | 0.020 | P2O5 | 0.331 | 0.020 |
| F | 0.214 | 0.020 | NiO | 0.211 | 0.010 | Na2O | 0.158 | 0.010 | F | 0.235 | 0.020 | NiO | 0.227 | 0.010 |
| Na2O | 0.210 | 0.010 | Na2O | 0.208 | 0.010 | NiO | 0.125 | 0.010 | Na2O | 0.219 | 0.010 | F | 0.218 | 0.020 |
| NiO | 0.193 | 0.010 | F | 0.192 | 0.020 | MnO | 0.071 | 0.008 | NiO | 0.166 | 0.010 | Na2O | 0.214 | 0.010 |
| BaO | 0.042 | 0.006 | Cr2O3 | 0.048 | 0.007 | SrO | 0.038 | 0.006 | Ne | 0.056 | 0.007 | ZrO2 | 0.095 | 0.009 |
| Cr2O3 | 0.035 | 0.006 | MnO | 0.026 | 0.005 | V2O5 | 0.032 | 0.005 | Cr2O3 | 0.035 | 0.006 | Cr2O3 | 0.047 | 0.006 |
| MnO | 0.024 | 0.005 | ZrO2 | 0.021 | 0.004 | Cr2O3 | 0.032 | 0.005 | BaO | 0.034 | 0.006 | MnO | 0.021 | 0.004 |
| ZrO2 | 0.021 | 0.004 | SrO | 0.015 | 0.004 | CuO | 0.026 | 0.005 | MnO | 0.027 | 0.005 | Cl | 0.013 | 0.003 |
| SrO | 0.013 | 0.003 | Cl | 0.013 | 0.003 | Cl | 0.024 | 0.005 | ZrO2 | 0.022 | 0.004 | CuO | 0.012 | 0.003 |
| CuO | 0.012 | 0.003 | ZnO | 0.012 | 0.003 | ZnO | 0.023 | 0.005 | SrO | 0.015 | 0.004 | SrO | 0.010 | 0.003 |
| Cl | 0.012 | 0.003 | Rb2O | 0.011 | 0.003 | ZrO2 | 0.021 | 0.004 | ZnO | 0.011 | 0.003 | ZnO | 0.009 | 0.003 |
| ZnO | 0.011 | 0.003 | PbO | 0.005 | 0.002 | Rb2O | 0.012 | 0.003 | Cl | 0.010 | 0.003 | Rb2O | 0.007 | 0.002 |

|  | 21 |  |  | 22 |  |  | 23 |  |  | 24 |  |  | 25 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 69.514 | 0.100 | SiO2 | 73.149 | 0.100 | SiO2 | 74.021 | 0.100 | SiO2 | 73.316 | 0.100 | SiO2 | 72.109 | 0.100 |
| Al2O3 | 12.046 | 0.100 | Al2O3 | 10.496 | 0.090 | Al2O3 | 8.980 | 0.090 | Al2O3 | 9.900 | 0.090 | Al2O3 | 10.509 | 0.090 |
| CaO | 7.350 | 0.080 | CaO | 5.817 | 0.070 | CaO | 7.258 | 0.080 | CaO | 6.633 | 0.070 | CaO | 6.474 | 0.070 |
| MgO | 4.088 | 0.060 | MgO | 3.905 | 0.060 | MgO | 3.677 | 0.060 | MgO | 3.762 | 0.060 | MgO | 3.750 | 0.060 |
| Fe2O3 | 2.855 | 0.050 | Fe2O3 | 2.713 | 0.050 | Fe2O3 | 2.503 | 0.050 | Fe2O3 | 2.793 | 0.050 | Fe2O3 | 3.251 | 0.050 |
| K2O | 1.812 | 0.040 | K2O | 1.587 | 0.040 | K2O | 1.345 | 0.030 | K2O | 1.488 | 0.040 | K2O | 1.533 | 0.040 |
| SO3 | 0.694 | 0.020 | SO3 | 0.742 | 0.030 | SO3 | 0.888 | 0.030 | SO3 | 0.622 | 0.020 | SO3 | 0.629 | 0.020 |
| TiO2 | 0.501 | 0.020 | TiO2 | 0.437 | 0.020 | TiO2 | 0.377 | 0.020 | TiO2 | 0.425 | 0.020 | TiO2 | 0.460 | 0.020 |
| P2O5 | 0.351 | 0.020 | P2O5 | 0.304 | 0.020 | P2O5 | 0.287 | 0.020 | P2O5 | 0.324 | 0.020 | NiO | 0.339 | 0.020 |
| Na2O | 0.233 | 0.010 | NiO | 0.254 | 0.020 | NiO | 0.259 | 0.020 | NiO | 0.219 | 0.010 | P2O5 | 0.312 | 0.020 |
| F | 0.198 | 0.020 | Na2O | 0.227 | 0.010 | Na2O | 0.205 | 0.010 | Na2O | 0.211 | 0.010 | Na2O | 0.217 | 0.010 |
| NiO | 0.187 | 0.010 | F | 0.183 | 0.020 | Cr2O3 | 0.052 | 0.007 | F | 0.159 | 0.010 | F | 0.207 | 0.020 |
| Cr2O3 | 0.046 | 0.006 | Cr2O3 | 0.045 | 0.006 | ZrO2 | 0.035 | 0.006 | Cr2O3 | 0.044 | 0.006 | Cr2O3 | 0.059 | 0.007 |
| ZrO2 | 0.037 | 0.006 | ZrO2 | 0.042 | 0.006 | MnO | 0.024 | 0.005 | ZrO2 | 0.038 | 0.006 | CuO | 0.038 | 0.006 |
| MnO | 0.026 | 0.005 | MnO | 0.024 | 0.005 | CeO2 | 0.022 | 0.004 | MnO | 0.022 | 0.004 | ZrO2 | 0.036 | 0.006 |
| SrO | 0.015 | 0.004 | CeO2 | 0.020 | 0.004 | Cl | 0.013 | 0.003 | SrO | 0.011 | 0.003 | MnO | 0.022 | 0.004 |
| Cl | 0.012 | 0.003 | Cl | 0.015 | 0.004 | SrO | 0.012 | 0.003 | Cl | 0.011 | 0.003 | Cl | 0.020 | 0.004 |
| CuO | 0.011 | 0.003 | SrO | 0.010 | 0.003 | V2O5 | 0.011 | 0.003 | ZnO | 0.007 | 0.002 | SrO | 0.011 | 0.003 |
| Rb2O | 0.008 | 0.003 | CuO | 0.008 | 0.003 | CuO | 0.011 | 0.003 | Rb2O | 0.006 | 0.002 | ZnO | 0.008 | 0.003 |
| ZnO | 0.008 | 0.003 | Rb2O | 0.007 | 0.003 | ZnO | 0.008 | 0.003 | CuO | 0.006 | 0.002 | Rb2O | 0.007 | 0.003 |

|  | 26 |  |  | 27 |  |  | 28 |  |  | 29 |  |  | 30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 70.649 | 0.100 | SiO2 | 68.600 | 0.100 | SiO2 | 70.308 | 0.100 | SiO2 | 65.809 | 0.100 | SiO2 | 63.403 | 0.100 |
| Al2O3 | 10.967 | 0.090 | Al2O3 | 13.036 | 0.100 | Al2O3 | 12.181 | 0.100 | Al2O3 | 11.473 | 0.100 | Al2O3 | 14.727 | 0.100 |
| CaO | 8.216 | 0.080 | CaO | 6.788 | 0.080 | CaO | 6.545 | 0.070 | CaO | 9.720 | 0.090 | CaO | 8.766 | 0.080 |
| MgO | 4.057 | 0.060 | MgO | 4.099 | 0.060 | MgO | 4.188 | 0.060 | MgO | 4.322 | 0.060 | MgO | 5.127 | 0.070 |
| Fe2O3 | 2.686 | 0.050 | Fe2O3 | 3.273 | 0.050 | Fe2O3 | 2.927 | 0.050 | Fe2O3 | 3.839 | 0.060 | Fe2O3 | 3.582 | 0.060 |
| K2O | 1.661 | 0.040 | K2O | 1.987 | 0.040 | K2O | 1.849 | 0.040 | K2O | 1.792 | 0.040 | K2O | 2.199 | 0.040 |
| TiO2 | 0.499 | 0.020 | TiO2 | 0.566 | 0.020 | TiO2 | 0.543 | 0.020 | SO3 | 1.259 | 0.030 | TiO2 | 0.608 | 0.020 |
| P2O5 | 0.340 | 0.020 | NiO | 0.394 | 0.020 | P2O5 | 0.341 | 0.020 | TiO2 | 0.535 | 0.020 | SO3 | 0.518 | 0.020 |
| Na2O | 0.249 | 0.010 | P2O5 | 0.354 | 0.020 | Na2O | 0.298 | 0.020 | P2O5 | 0.336 | 0.020 | P2O5 | 0.337 | 0.020 |
| F | 0.207 | 0.020 | Na2O | 0.263 | 0.020 | SO3 | 0.268 | 0.020 | Na2O | 0.280 | 0.020 | Na2O | 0.281 | 0.020 |
| NiO | 0.160 | 0.010 | SO3 | 0.239 | 0.010 | NiO | 0.204 | 0.010 | NiO | 0.248 | 0.010 | F | 0.210 | 0.020 |
| SO3 | 0.139 | 0.010 | F | 0.187 | 0.020 | F | 0.174 | 0.020 | F | 0.180 | 0.020 | NiO | 0.084 | 0.009 |
| Cr2O3 | 0.039 | 0.006 | Cr2O3 | 0.076 | 0.008 | Cr2O3 | 0.044 | 0.006 | Cr2O3 | 0.050 | 0.007 | MnO | 0.032 | 0.005 |
| MnO | 0.033 | 0.005 | MnO | 0.029 | 0.005 | ZrO2 | 0.029 | 0.005 | MnO | 0.043 | 0.006 | Cl | 0.030 | 0.005 |
| ZrO2 | 0.025 | 0.005 | ZrO2 | 0.026 | 0.005 | MnO | 0.028 | 0.005 | ZrO2 | 0.024 | 0.005 | Cr2O3 | 0.023 | 0.005 |
| CuO | 0.023 | 0.005 | SrO | 0.015 | 0.004 | Cl | 0.020 | 0.004 | Cl | 0.021 | 0.004 | ZrO2 | 0.021 | 0.004 |
| SrO | 0.014 | 0.004 | Nd2O3 | 0.014 | 0.004 | SrO | 0.014 | 0.004 | SrO | 0.016 | 0.004 | SrO | 0.016 | 0.004 |
| Cl | 0.012 | 0.003 | Cl | 0.013 | 0.003 | CuO | 0.012 | 0.003 | ZnO | 0.015 | 0.004 | ZnO | 0.011 | 0.003 |
| ZnO | 0.009 | 0.003 | CuO | 0.012 | 0.003 | ZnO | 0.011 | 0.003 | CuO | 0.014 | 0.004 | Rb2O | 0.010 | 0.003 |
| Rb2O | 0.007 | 0.003 | Rb2O | 0.010 | 0.003 | Rb2O | 0.008 | 0.003 | Rb2O | 0.009 | 0.003 |  |  |  |

|  | 31 |  |  | 32 |  |  | 33 |  |  | 34 |  |  | 35 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 68.824 | 0.100 | SiO2 | 70.493 | 0.100 | SiO2 | 70.505 | 0.100 | SiO2 | 70.036 | 0.100 | SiO2 | 66.243 | 0.100 |
| Al2O3 | 14.705 | 0.100 | Al2O3 | 13.379 | 0.100 | Al2O3 | 12.251 | 0.100 | Al2O3 | 13.524 | 0.100 | Al2O3 | 15.726 | 0.100 |
| MgO | 4.654 | 0.060 | MgO | 4.762 | 0.060 | CaO | 4.621 | 0.060 | CaO | 5.155 | 0.070 | CaO | 5.298 | 0.070 |
| CaO | 3.652 | 0.060 | CaO | 4.619 | 0.060 | MgO | 4.484 | 0.060 | MgO | 4.373 | 0.060 | MgO | 4.625 | 0.060 |
| Fe2O3 | 3.333 | 0.050 | Fe2O3 | 3.023 | 0.050 | Fe2O3 | 3.246 | 0.050 | Fe2O3 | 3.109 | 0.050 | Fe2O3 | 3.583 | 0.060 |
| K2O | 2.043 | 0.040 | K2O | 1.912 | 0.040 | K2O | 1.855 | 0.040 | K2O | 1.946 | 0.040 | K2O | 2.239 | 0.040 |
| SO3 | 1.106 | 0.030 | TiO2 | 0.497 | 0.020 | SO3 | 1.343 | 0.030 | TiO2 | 0.544 | 0.020 | TiO2 | 0.605 | 0.020 |
| TiO2 | 0.578 | 0.020 | P2O5 | 0.303 | 0.020 | TiO2 | 0.523 | 0.020 | P2O5 | 0.352 | 0.020 | SO3 | 0.496 | 0.020 |
| P2O5 | 0.304 | 0.020 | Na2O | 0.267 | 0.020 | Na2O | 0.302 | 0.020 | Na2O | 0.272 | 0.020 | P2O5 | 0.330 | 0.020 |
| Na2O | 0.287 | 0.020 | F | 0.235 | 0.020 | P2O5 | 0.300 | 0.020 | F | 0.218 | 0.020 | Na2O | 0.282 | 0.020 |
| F | 0.198 | 0.020 | NiO | 0.185 | 0.010 | NiO | 0.221 | 0.010 | SO3 | 0.161 | 0.010 | F | 0.266 | 0.020 |
| NiO | 0.151 | 0.010 | SO3 | 0.155 | 0.010 | F | 0.191 | 0.020 | NiO | 0.136 | 0.010 | NiO | 0.106 | 0.010 |
| Cl | 0.036 | 0.006 | Cr2O3 | 0.034 | 0.006 | Cr2O3 | 0.042 | 0.006 | Cr2O3 | 0.034 | 0.006 | Cl | 0.034 | 0.006 |
| Cr2O3 | 0.031 | 0.005 | Cl | 0.032 | 0.005 | Cl | 0.032 | 0.005 | Cl | 0.030 | 0.005 | MnO | 0.034 | 0.006 |
| ZrO2 | 0.028 | 0.005 | ZrO2 | 0.027 | 0.005 | ZrO2 | 0.031 | 0.005 | ZrO2 | 0.028 | 0.005 | CeO2 | 0.034 | 0.006 |
| MnO | 0.018 | 0.004 | MnO | 0.022 | 0.004 | SrO | 0.012 | 0.003 | MnO | 0.023 | 0.005 | Cr2O3 | 0.030 | 0.005 |
| ZnO | 0.013 | 0.003 | Dy2O3 | 0.014 | 0.004 | MnO | 0.011 | 0.003 | CuO | 0.015 | 0.004 | ZrO2 | 0.021 | 0.004 |
| SrO | 0.011 | 0.003 | SrO | 0.011 | 0.003 | CuO | 0.009 | 0.003 | ZnO | 0.013 | 0.003 | SrO | 0.014 | 0.004 |
| Rb2O | 0.010 | 0.003 | Rb2O | 0.008 | 0.003 | Rb2O | 0.009 | 0.003 | SrO | 0.012 | 0.003 | ZnO | 0.011 | 0.003 |
| CuO | 0.009 | 0.003 | ZnO | 0.007 | 0.002 | ZnO | 0.008 | 0.003 | Rb2O | 0.009 | 0.003 | Rb2O | 0.010 | 0.003 |

|  | 36 |  |  | 37 |  |  | 38 |  |  | 39 |  |  | 40 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 74.142 | 0.100 | SiO2 | 69.927 | 0.100 | SiO2 | 71.462 | 0.100 | SiO2 | 73.200 | 0.100 | SiO2 | 68.997 | 0.100 |
| Al2O3 | 11.470 | 0.100 | Al2O3 | 12.102 | 0.100 | Al2O3 | 11.268 | 0.090 | Al2O3 | 9.203 | 0.090 | Al2O3 | 12.222 | 0.100 |
| MgO | 3.922 | 0.060 | CaO | 6.168 | 0.070 | CaO | 6.070 | 0.070 | CaO | 7.653 | 0.080 | CaO | 6.775 | 0.080 |
| CaO | 3.278 | 0.050 | MgO | 4.363 | 0.060 | MgO | 4.344 | 0.060 | MgO | 3.868 | 0.060 | MgO | 4.644 | 0.060 |
| Fe2O3 | 2.903 | 0.050 | Fe2O3 | 2.999 | 0.050 | Fe2O3 | 2.860 | 0.050 | Fe2O3 | 2.640 | 0.050 | Fe2O3 | 2.902 | 0.050 |
| K2O | 1.639 | 0.040 | K2O | 1.785 | 0.040 | K2O | 1.669 | 0.040 | K2O | 1.470 | 0.040 | K2O | 1.872 | 0.040 |
| SO3 | 0.777 | 0.030 | SO3 | 0.792 | 0.030 | SO3 | 0.576 | 0.020 | TiO2 | 0.419 | 0.020 | SO3 | 0.905 | 0.030 |
| TiO2 | 0.495 | 0.020 | TiO2 | 0.473 | 0.020 | TiO2 | 0.494 | 0.020 | SO3 | 0.354 | 0.020 | TiO2 | 0.481 | 0.020 |
| P2O5 | 0.346 | 0.020 | Na2O | 0.340 | 0.020 | Na2O | 0.335 | 0.020 | P2O5 | 0.288 | 0.020 | Na2O | 0.419 | 0.020 |
| Na2O | 0.289 | 0.020 | P2O5 | 0.331 | 0.020 | P2O5 | 0.324 | 0.020 | NiO | 0.275 | 0.020 | P2O5 | 0.324 | 0.020 |
| NiO | 0.247 | 0.010 | NiO | 0.284 | 0.020 | NiO | 0.249 | 0.010 | Na2O | 0.268 | 0.020 | NiO | 0.285 | 0.020 |
| F | 0.195 | 0.020 | F | 0.195 | 0.020 | F | 0.163 | 0.010 | F | 0.169 | 0.020 | Cr2O3 | 0.058 | 0.007 |
| ZrO2 | 0.149 | 0.010 | Cr2O3 | 0.057 | 0.007 | Cr2O3 | 0.047 | 0.007 | Cr2O3 | 0.053 | 0.007 | ZrO2 | 0.033 | 0.005 |
| Cr2O3 | 0.051 | 0.007 | Cl | 0.047 | 0.007 | ZrO2 | 0.035 | 0.006 | MnO | 0.030 | 0.005 | MnO | 0.020 | 0.004 |
| Cl | 0.023 | 0.005 | ZrO2 | 0.037 | 0.006 | MnO | 0.022 | 0.004 | ZrO2 | 0.028 | 0.005 | Cl | 0.018 | 0.004 |
| CeO2 | 0.019 | 0.004 | MnO | 0.026 | 0.005 | Cl | 0.022 | 0.004 | CeO2 | 0.026 | 0.005 | SrO | 0.013 | 0.003 |
| MnO | 0.018 | 0.004 | CeO2 | 0.020 | 0.004 | ZnO | 0.016 | 0.004 | Cl | 0.014 | 0.004 | Rb2O | 0.009 | 0.003 |
| SrO | 0.009 | 0.003 | SrO | 0.012 | 0.003 | CuO | 0.015 | 0.004 | CuO | 0.012 | 0.003 | ZnO | 0.008 | 0.003 |
| ZnO | 0.008 | 0.003 | CuO | 0.011 | 0.003 | SrO | 0.011 | 0.003 | SrO | 0.012 | 0.003 | CuO | 0.007 | 0.002 |
| CuO | 0.007 | 0.002 | ZnO | 0.010 | 0.003 | Rb2O | 0.007 | 0.002 | ZnO | 0.008 | 0.003 | PbO | 0.005 | 0.002 |

|  | 41 |  |  | 42 |  |  | 43 |  |  | 44 |  |  | CL |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SiO2 | 74.142 | 0.100 | SiO2 | 69.927 | 0.100 | SiO2 | 71.462 | 0.100 | SiO2 | 73.200 | 0.100 | SiO2 | 68.997 | 0.100 |
| Al2O3 | 11.470 | 0.100 | Al2O3 | 12.102 | 0.100 | Al2O3 | 11.268 | 0.090 | Al2O3 | 9.203 | 0.090 | Al2O3 | 12.222 | 0.100 |
| MgO | 3.922 | 0.060 | CaO | 6.168 | 0.070 | CaO | 6.070 | 0.070 | CaO | 7.653 | 0.080 | CaO | 6.775 | 0.080 |
| CaO | 3.278 | 0.050 | MgO | 4.363 | 0.060 | MgO | 4.344 | 0.060 | MgO | 3.868 | 0.060 | MgO | 4.644 | 0.060 |
| Fe2O3 | 2.903 | 0.050 | Fe2O3 | 2.999 | 0.050 | Fe2O3 | 2.860 | 0.050 | Fe2O3 | 2.640 | 0.050 | Fe2O3 | 2.902 | 0.050 |
| K2O | 1.639 | 0.040 | K2O | 1.785 | 0.040 | K2O | 1.669 | 0.040 | K2O | 1.470 | 0.040 | K2O | 1.872 | 0.040 |
| SO3 | 0.777 | 0.030 | SO3 | 0.792 | 0.030 | SO3 | 0.576 | 0.020 | TiO2 | 0.419 | 0.020 | SO3 | 0.905 | 0.030 |
| TiO2 | 0.495 | 0.020 | TiO2 | 0.473 | 0.020 | TiO2 | 0.494 | 0.020 | SO3 | 0.354 | 0.020 | TiO2 | 0.481 | 0.020 |
| P2O5 | 0.346 | 0.020 | Na2O | 0.340 | 0.020 | Na2O | 0.335 | 0.020 | P2O5 | 0.288 | 0.020 | Na2O | 0.419 | 0.020 |
| Na2O | 0.289 | 0.020 | P2O5 | 0.331 | 0.020 | P2O5 | 0.324 | 0.020 | NiO | 0.275 | 0.020 | P2O5 | 0.324 | 0.020 |
| NiO | 0.247 | 0.010 | NiO | 0.284 | 0.020 | NiO | 0.249 | 0.010 | Na2O | 0.268 | 0.020 | NiO | 0.285 | 0.020 |
| F | 0.195 | 0.020 | F | 0.195 | 0.020 | F | 0.163 | 0.010 | F | 0.169 | 0.020 | Cr2O3 | 0.058 | 0.007 |
| ZrO2 | 0.149 | 0.010 | Cr2O3 | 0.057 | 0.007 | Cr2O3 | 0.047 | 0.007 | Cr2O3 | 0.053 | 0.007 | ZrO2 | 0.033 | 0.005 |
| Cr2O3 | 0.051 | 0.007 | Cl | 0.047 | 0.007 | ZrO2 | 0.035 | 0.006 | MnO | 0.030 | 0.005 | MnO | 0.020 | 0.004 |
| Cl | 0.023 | 0.005 | ZrO2 | 0.037 | 0.006 | MnO | 0.022 | 0.004 | ZrO2 | 0.028 | 0.005 | Cl | 0.018 | 0.004 |
| CeO2 | 0.019 | 0.004 | MnO | 0.026 | 0.005 | Cl | 0.022 | 0.004 | CeO2 | 0.026 | 0.005 | SrO | 0.013 | 0.003 |
| MnO | 0.018 | 0.004 | CeO2 | 0.020 | 0.004 | ZnO | 0.016 | 0.004 | Cl | 0.014 | 0.004 | Rb2O | 0.009 | 0.003 |
| SrO | 0.009 | 0.003 | SrO | 0.012 | 0.003 | CuO | 0.015 | 0.004 | CuO | 0.012 | 0.003 | ZnO | 0.008 | 0.003 |
| ZnO | 0.008 | 0.003 | CuO | 0.011 | 0.003 | SrO | 0.011 | 0.003 | SrO | 0.012 | 0.003 | CuO | 0.007 | 0.002 |
| CuO | 0.007 | 0.002 | ZnO | 0.010 | 0.003 | Rb2O | 0.007 | 0.002 | ZnO | 0.008 | 0.003 | PbO | 0.005 | 0.002 |

Table A2: thin-sections image analysis results

|  | Grain size fraction area % | Particle diameter (mm) | Color threshold | Particle threshold |
| --- | --- | --- | --- | --- |
| Sample | 3,243 | 0,050 | 80 | high |
| 5 | 4,744 | 0,023 | 80 | low |
| Thin section | 17,551 | 0,022 | 30 | low |
| 1 | 11,208 | 0,061 | 30 | high |
| Sample | 4,583 | 0,050 | 150 | high |
| 5 | 6,196 | 0,030 | 150 | low |
| Thin section | 19,122 | 0,030 | 50 | low |
| 2 | 15,989 | 0,064 | 50 | high |
| Sample | 0,591 | 0,018 | 80 | low |
| 14 | 0,238 | 0,034 | 80 | high |
| Thin section | 0,488 | 0,042 | 50 | high |
| 1 | 1,463 | 0,017 | 50 | low |
| Sample | 0,785 | 0,017 | 80 | low |
| 14 | 0,211 | 0,031 | 80 | high |
| Thin section | 0,348 | 0,034 | 50 | high |
| 2 | 1,928 | 0,017 | 50 | low |
| Sample | 9,490 | 0,057 | 170 | high |
| 23 | 11,069 | 0,033 | 170 | low |
| Thin section | 37,938 | 0,042 | 45 | low |
| 1 | 35,655 | 0,086 | 45 | high |
| Sample | 6,293 | 0,045 | 120 | high |
| 23 | 9,311 | 0,025 | 120 | low |
| Thin section | 35,097 | 0,030 | 40 | low |
| 2 | 28,964 | 0,072 | 40 | high |
| Sample | 0,851 | 0,016 | 70 | low |
| 30 | 0,064 | 0,038 | 70 | high |
| Thin section | 0,297 | 0,039 | 45 | high |
| 1 | 2,356 | 0,018 | 45 | low |
| Sample | 1,945 | 0,017 | 90 | low |
| 30 | 0,381 | 0,037 | 90 | high |
| Thin section | 1,068 | 0,043 | 60 | high |
| 2 | 3,864 | 0,017 | 60 | low |
| Sample | 0,648 | 0,041 | 90 | high |
| 40 | 3,161 | 0,018 | 90 | low |
| Thin section | 8,896 | 0,023 | 50 | low |
| 1 | 4,301 | 0,046 | 50 | high |
| Sample | 0,656 | 0,039 | 90 | high |
| 40 | 2,798 | 0,019 | 90 | low |
| Thin section | 6,316 | 0,021 | 50 | low |
| 2 | 2,278 | 0,043 | 50 | high |

Table A3: CT SCAN image analysis results

|  | Grain size fraction area % | Particle diameter (mm) | Color threshold | Particle threshold |
| --- | --- | --- | --- | --- |
| CT5a | 6,122 | 0,024 | 8383 | low |
| CT5b | 4,216 | 0,048 | 8383 | high |
| CT5c | 23,033 | 0,036 | 7893 | low |
| CT5d | 20,229 | 0,072 | 7893 | high |
| CT14b | 0,466 | 0,045 | 11159 | high |
| CT14c | 4,430 | 0,020 | 10490 | low |
| CT14d | 1,542 | 0,046 | 10490 | high |
| CT23a | 7,79 | 0,023 | 6675 | low |
| CT23b | 4,5255 | 0,049 | 6675 | high |
| CT23c | 20,828 | 0,032 | 6231 | low |
| CT23D | 17,104 | 0,068 | 6231 | high |
| CT30a | 4,6413 | 0,019 | 7787 | low |
| CT30b | 1,2852 | 0,041 | 7787 | high |
| CT30c | 10,438 | 0,022 | 7470 | low |
| CT30d | 4,5983 | 0,047 | 7470 | high |
| CT40a | 5,483 | 0,021 | 7670 | low |
| CT40b | 2,407 | 0,045 | 7670 | high |
| CT40c | 18,81 | 0,028 | 7172 | low |
| CT40d | 14,016 | 0,064 | 7172 | high |